SECTION 11 66 23. 23 [11483]

GYMNASIUM VOLLEYBALL SYSTEMS

***** Draper, Inc. manufactures floor sleeves and anchors, volleyball and other game standards, nets, judges stand, and other accessories required for competition type volleyball installations within gymnasiums. Three types of standards are available - telescoping, fixed length, and portable. Various components, accessories, and options are listed in this specification section. The specifier will need to select the appropriate items and delete non-applicable components and options. *****

PART 1 - GENERAL

1.1	SUMMARY

A.	Section includes: Volleyball equipment for gymnasiums and other interior installations:

***** Edit this list to indicate items being specified. *****

1.	Floor sleeves and cover plates.

2.	Floor anchors.

3.	[Telescoping] [Fixed length] [Portable] standards.

4.	Volleyball nets, rope tensioners, boundary markers, and antennae.

5.	Judges stands.

6.	Protective pads.

7.	Volleyball equipment transporters and wall storage hooks.

B.	Related sections:

***** List other specification sections dealing with work directly related to this section such as the following. *****

1.	Section 03300 - Cast-in-Place Concrete: Concrete floor slabs and footings to receive floor sleeves.

2.	Section 09640 - Wood Flooring: Layout and painting of court lines to be coordinated with installation of floor [sleeves] [anchors].

3.	Section 09650 - Resilient Flooring: Layout of court lines to be coordinated with installation of floor [sleeves] [anchors].

1.2	REFERENCES

***** List by number and full title reference standards referred to in remainder of the specification section. Delete non-applicable references. *****

A.	ASTM A500 - Formed Welded Seamless Structural Tubing in Rounds and Shapes.

B.	ASTM B85 - Aluminum-Alloy Die Castings.

C.	ASTM B221 - Aluminum-Alloy Extruded Bar, Rod, Wire, Shape, and Tube.

1.3	SUBMITTALS

A.	Submit in accordance with Section 01330 - Submittal Procedures:

1.	List of proposed products and product data.

2.	Shop drawings showing volleyball court layout and floor [sleeve] [anchor] locations, dimensions, and method of installation.

3.	Manufacturer's installation instructions.

1.4	QUALITY ASSURANCE

A.	All volleyball equipment, components, and accessories shall be products of a single manufacturer.

B.	Volleyball equipment shall be designed, fabricated, and installed to comply with requirements for competition play of the following associations:

1.	Federation Internationale de Volleyball (FIVB).

2.	National Collegiate Athletic Association (NCAA).

3.	National Federation of State High School Associations (NFHS).

4.	USA Volleyball (USAV).

PARTS 2 - PRODUCTS

2.1	ACCEPTABLE MANUFACTURERS

A.	Draper, Inc., 411 South Pearl Street, Spiceland, Indiana 47385-0425; 765-987-7999.

B.	Manufacturers of equivalent products submitted and approved in accordance with Section 01630 - Product Substitution Procedures.

2.2	MATERIALS

A.	Structural steel tubing: Steel, mechanical, round tubing conforming to ASTM A500.

B.	Extruded aluminum tubing: Schedule 80, ASTM B221, alloy 6063 Temper T5.

C.	Aluminum castings: ASTM B85.

2.3	CEILING SUSPENDED VOLLEYBALL SYSTEMS

**** Draper Inc. manufacturers a complete overhead volleyball system that is ceiling suspended and electrically operated. Use this article to specify Ceiling Suspended System. *****

A.	Type: Overhead volleyball system shall be two independent, welded support frames constructed from steel mechanical tubing.

B.	Support Frame: Each main support frame shall have 4” OD 11-guage front steel tubing and formed 1- ½” OD 11-gauge back brace with ¼: x 2” HR steel Intermediate support truss supports. Each support frame shall have stabilizing foot consisting of 3 ½” OD aluminum telescopic inner tubes that are lowered and locked into position using internal threaded rod and gear assemblies. Stabilizing foot shall be designed to provide superior rigidity and net tension. The bottom inner tube is provided with a special non-marking nylon spherical shaped nylon end pad to protect and minimize contact with floor should post be folded without feet being raised. The main support frame shall pivot on 1¼" minimum solid steel shaft secured in a milled bearing hole in ½" minimum steel plate hangers to insure accurate positioning.

C.	Folding Brace: The folding side braces shall be jackknife type, fully adjustable, self-locking in the down position and constructed of 2½" OD 13-gauge (outer) steel tubing and 2¼" OD 14-gauge (inner) steel tubing. The volleyball system shall be supported from 4" OD 11-gauge steel mechanical tubing (as shown on plans) anchored to roof structure by means of heavy steel support hangers. Superstructure tubes shall be reinforced with bridging and/or bracing when truss centers exceed 12'0". Support frame, folding braces and overhead superstructure shall have a black or white powder coat finish. Optional colors are available.

D;	Operation: Volleyball system is raised or lowered by a ¼" (6.35 mm) aircraft cable, certified minimum break strength of 7,000 pounds (3178 kg), operating over aluminum alloy sheaves with bronze bearings that do not require lubrication. Cable is retracted by a special dual drum electric winch.

E:	Net Attachment: Each support frame shall have a sliding net attachment post to allow infinite height adjustment from 6' (1.854 m) to 8'4" (2.540 cm) to meet all age group height settings from elementary school use to international competition for both men and women. Posts shall incorporate a threaded rod mechanism for easy, precise and infinite height adjustments. Net attachment post shall have a powder coat finish to match folding frame.

F:	Net Tensioning Winch: One net attachment post shall incorporate a heavy-duty, self-locking worm gear mechanism (Net winch). Net winch shall be furnished with a heavy 2" (5 cm) wide high tensile nylon strap with heavy-duty snap hook to eliminate the possibility of hook breaking and guarantee safe connection to net top cable. The winch is furnished with a folding handle for player safety.

G.	Volleyball Net: Fabricated from high quality [4 inches] [102 mm] square mesh made of 2.5 mm black knotless nylon; Net 500014 as manufactured by Draper, Inc.

1.	Size: [32 feet] [9.8 m] long by [39-3/8 inches] [1000 mm] high.

2.	Provide double stitched, vinyl coated polyester hem around perimeter of net. At net ends, provide hem with pocket containing [1/2 inch] [133 mm] diameter fiberglass dowel.

3.	Net cable: [1/8 inch] [3 mm] diameter, [2000 pounds] [907 kg] minimum breaking strength, galvanized aircraft cable with nylon coating. Equip ends with loops formed with heavy swaged type fittings. Run cable through top hem.

4.	Rope tensioner: Provide bottom hem with [1/4 inch] [6 mm] braided nylon rope and ratchet style tensioner.

5.	Provide each net end with three [1 inch] [25 mm] wide polypropylene tension straps with buckles for tightening net.

6.	Combination Boundary Markers and Antennae: Pair of [2 inches] [51 mm] wide, white, polyester reinforced vinyl strips to attach with snap fasteners to ends of net at boundary lines; Pair of [3/8 inch] [100 mm] diameter fiberglass rods with red and white strips to extend above net [32-1/2 inches] [826mm]; 500016 Boundary Markers and Antenna as manufactured by Draper, Inc

I.	Protection Pads: Removable, rectangular protection pads with hinged corners to be attached to standards to protect players

1.	Sizes: Designed to fit around standard with tensioning winch: [6 feet] [1829 mm] high by [10-1/2 inches] [267 mm] by [6-1/2 inches] [165 mm].

2.	Foam: [1-1/4 inches] [32 mm] thick Ethafoam.

3.	Cover: [14 ounces] [397 grams], polyester reinforced vinyl.

4.	Vinyl color: [Dark blue] [Marine blue] [White] [Red] [Beige] [Orange] [Yellow] [Gray] [Maroon] [Black] [Purple] [Forest green] [Kelly Green] [Navy

***** Include the following to include optional Judges Stand for ceiling suspended volleyball system. *****

J.	Judges Stand: Welded and mechanically attached with a welded frame from [1-1/2 inches] [938 mm] square [14-gauge] [2 mm] square tubing equipped with a varnished birch plywood platform. Judges stand provided with vinyl covered foam protection pads. Vinyl color shall be [Dark Blue] [Marine Blue] [White] [Red] [Beige] [Orange] [Yellow] [Gray] [Maroon] [Black] [Purple] [Forest Green] [Kelly Green] [Navy Blue].

2.4	FLOOR SLEEVES AND COVERS

***** Draper, Inc. provides floor sleeves and cover plates to accommodate telescoping and fixed length game standards. 3-1/2 inches (89 mm) diameter sleeves are required for telescoping and fixed length volleyball standards. Draper, Inc. also manufactures 2-3/8, 3, and 4 inches (60, 76, and 102 mm) diameter sleeves. *****

A.	Floor sleeve: Round, mechanical steel tube sleeve welded to steel anchor plate for casting in concrete floor to receive volleyball standard; Floor Sleeve 501006 as manufactured by Draper, Inc.

B.	Size: [3-1/2 inches] [89 mm] inside diameter by [8-1/2 inches] [216 mm] tube and [4 inches] [102 mm] square anchor plate.

***** Draper, Inc. manufactures 5 cover plates to be used with floor sleeves. *****

*************************************************************************

***** Use the following paragraph to specify chrome plated Cover Plate 501001. *****

C.	Cover plate: Cast aluminum recessed mounting flange and [5 inches] [127 mm] diameter chrome plated cover; Cover Plate 501001 as manufactured by Draper, Inc.

1.	Equip cover with swivel type retaining screw and key to open and lock cover.

2.	Cover installed with flat head wood screws.

**********************************************************************

***** Use the following paragraph to specify brass plated Cover Plate 501031. *****

C.	Cover plate: [5 inches] [127 mm] brass plated floor plate recessed mounting flange; Cover Plate 501031 as manufactured by Draper, Inc.

1.	Cover installed with flat head wood screws.

2.	Opening diameter: [4-1/8 inches] [108 mm].

****************************************************************************

***** Use the following paragraph to specify locking flanged brass Cover Plate 501035. *****

C.	Cover plate: [7-1/2 inches] [190 mm] brass floor plate mounting flange; Cover Plate 501035 as manufactured by Draper, Inc.

- Cover installed with flat head wood screws.

- Locking mechanism to prevent bouncing of cover plate.

- Opening diameter: [4-3/8 inches] [108 mm].

****************************************************************************

***** Use the following paragraph to specify concealed brass plated Cover Plate 501033 with key for locking cover. *****

C.	Cover plate: [8 inches] [203 mm] solid brass floor plate and permanently attached hinged cover; Cover Plate 501033 as manufactured by Draper, Inc.

1.	Cover installed with flat head brass wood screws.

2.	Opening diameter: [7-3/16 inches] [183 mm].

3.	Mounting holes in floor plate completely concealed when cover is closed.

- Provide key for opening and locking cover.

*********************************************************************

***** Use the following paragraph to specify chrome plated Cover Plate 501034 which is designed for use with existing floors. *****

C.	Cover plate: Cast aluminum [8 inches] [203 mm] diameter chrome plated cover designed for use with existing floor; Cover Plate 501034 as manufactured by Draper, Inc.

1.	Equip cover with swivel type retaining screw and key to open and lock cover.

2.5	FLOOR ANCHORS

***** Draper, Inc. manufactures two types of floor anchor. Model 501011 is installed in wood floors over concrete substrate. Model 501012 is installed in concrete floors with and without resilient floor finish. Floor anchors are required for attaching bases of portable game standards Models PTVS-01. If portable game standards are not required, delete this article. *****

A.	Type: Floor anchor to be recessed in [wood flooring on concrete substrate] [concrete floor slab [with [resilient floor finish]] and used for securing portable game standards; Model [501011] [501012] Floor Anchor as manufactured by Draper, Inc.

B.	Construction: Chrome plated, one-piece anchor with spring-loaded flush fairings and adjustment bolt and nut and designed for installation in concrete slab with fast setting grout.

*********************************************************************

***** Include the following paragraph for Model 501011 Floor Anchor. *****

C.	Size: [4-11/16 inches] [121 mm] long with [3-1/2 inches] [89 mm] diameter floor plate provided with wood screws for anchoring to wood floor.

*********************************************************************

***** Include the following paragraph for Model 501012 Floor Anchor. *****

C.	Size: [4-11/16 inches] [121 mm] long by [1-1/2 inches] [89 mm] diameter.

2.6	TELESCOPING ALUMINUM STANDARDS

**** Draper Inc. manufacturers five types of game standards for supporting nets: telescoping, fixed length, and portable standards. Use this article to specify Telescoping Standards. *****

A.	Types: Telescoping standards fabricated from Schedule 80 aluminum bottom and upper tubes and capable of adjusting from [73 inches] [1854 mm] to [100 inches] [2540 mm] in [1 inch] [25 mm] increments to meet all age group height settings.

***** A pair of standards, PVS-01 [500001], is required for a single volleyball court. Installations with multiple courts can be achieved by using PVS-02 [500002] center standard system. *****

5.	Standard PVS-01 500001 as manufactured by Draper, Inc.: Pair of standards, one with tensioning winch and other with adjustable cable anchoring collar. Both standards equipped with single pulley sheave on upper telescoping tube section.

6.	Center Standard System 500002: 3 Post system, center standard equipped with two pulley sheaves on upper telescoping tube section spaced to accommodate two opposing net cables. Provide adjustable cable anchor collar to accommodate two nets.

B.	Bottom tube section: [3-1/2 inches] [89 mm] diameter with [0.30 inch] [8 mm] wall thickness, [72 inches] [1829 mm] high. Bottom provided with rubber foot to protect floors.

C.	Upper telescoping tube section: [2-7/8 inches] [73 mm] diameter with [0.28 inch] [7 mm] wall thickness.

D.	Pulley sheaves: [4 inches] [102 mm] diameter pulley and oilite bushing attached to top of upper telescoping tube.

E.	Tensioning winch: Heavy-duty, self-locking worm-gear mechanism.

1.	Position winch on outside of bottom tube.

2.	Equip winch with [2 inches] [51 mm] wide, high tensile nylon strap with sling ring and spring-hook for connection to net cable.

3.	Winch operated by folding handle.

2.7	TELESCOPING STEEL STANDARDS

**** Draper Inc. manufacturers five types of game standards for supporting nets. Use this article to specify Telescoping Steel Standards. *****

A.	Types: Telescoping standards fabricated from Schedule 80 aluminum upper tube and 11-gauge steel bottom tube and capable of adjusting from [73 inches] [1854 mm] to [100 inches] [2540 mm] in [1 inch] [25 mm] increments to meet all age group height settings.

***** A pair of standards, SVS-01 [500021], is required for a single volleyball court. Installations with multiple courts can be achieved by using SVS-02 [500022] center standard system. *****

4.	Standard SVS-01 500021 as manufactured by Draper, Inc.: Pair of standards, one with tensioning winch and other with adjustable cable anchoring collar. Both standards equipped with single pulley sheave on upper telescoping tube section.

5.	Center Standard System 500022: 3 Post system, center standard equipped with two pulley sheaves on upper telescoping tube section spaced to accommodate two opposing net cables. Provide adjustable cable anchor collar to accommodate two nets.

B.	Bottom tube section: [3-1/2 inches] [89 mm] diameter with 11 gauge steel, [0.120 inch] [3 mm] wall thickness, [72 inches] [1829 mm] high. Bottom provided with rubber foot to protect floors.

C.	Upper telescoping tube section: [2-7/8 inches] [73 mm] diameter with [0.28 inch] [7 mm] wall thickness.

D.	Pulley sheaves: [4 inches] [102 mm] diameter pulley and oilite bushing attached to top of upper telescoping tube.

E.	Tensioning winch: Heavy-duty, self-locking worm-gear mechanism.

1.	Position winch on outside of bottom tube.

2.	Equip winch with [2 inches] [51 mm] wide, high tensile nylon strap with sling ring and spring-hook for connection to net cable.

3.	Winch operated by folding handle.

2.8	SPRING ASSIST TELESCOPING ALUMINUM STANDARDS

**** Draper Inc. manufacturers five types of game standards for supporting nets. Use this article to specify Spring Assist Telescoping Standards. *****

A.	Types: Telescoping standards fabricated from Schedule 80 aluminum bottom and upper tubes and capable of adjusting from [73 inches] [1854 mm] to [100 inches] [2540 mm] in [1 inch] [25 mm] increments to meet all age group height settings.

***** A pair of standards, EVS-01 [500041], is required for a single volleyball court. Installations with multiple courts can be achieved by using EVS-02 [500042] center standard system. *****

4.	Standard EVS-01 500041 as manufactured by Draper, Inc.: Pair of standards, one with tensioning winch and other with adjustable cable anchoring collar. Both standards equipped with single pulley sheave on upper telescoping tube section. Spring assist with infinite height adjustment.

5.	Center Standard EVS-02 System 500042: 3 Post system, center standard equipped with two pulley sheaves on upper telescoping tube section spaced to accommodate two opposing net cables. Provide adjustable cable anchor collar to accommodate two nets. Spring assist with infinite height adjustment.

B.	Bottom tube section: [3-1/2 inches] [89 mm] diameter with [0.30 inch] [8 mm] wall thickness, [72 inches] [1829 mm] high. Bottom provided with rubber foot to protect floors.

C.	Upper telescoping tube section: [2-7/8 inches] [73 mm] diameter with [0.28 inch] [7 mm] wall thickness.

D.	Pulley sheaves: [4 inches] [102 mm] diameter pulley and oilite bushing attached to top of upper telescoping tube.

E.	Tensioning winch: Heavy-duty, self-locking worm-gear mechanism.

1.	Position winch on outside of bottom tube.

2.	Equip winch with [2 inches] [51 mm] wide, high tensile nylon strap with sling ring and spring-hook for connection to net cable.

3.	Winch operated by folding handle.

2.9	FIXED LENGTH STANDARDS

***** Use this article to specify CVS-01 500031 Fixed Length Standard. *****

A.	Type: Pair of fixed length game standards fabricated from aluminum tubing and designed for installation in floor sleeves; CVS-01 500031 as manufactured by Draper, Inc.

B.	Construction: [3-1/2 inches] [89 mm] diameter extruded 6063-T6 aluminum with clear anodized finish. Upright is custom designed to reduce weight while providing excellent rigidity. Wall Thickness if upright tube varies from [0.216 inches] [5.5 mm] to [0.433 inches] [11 mm] with thicker walls positioned to reduce deflection. Post features an indexing channel to prevent sliding collard from rotating and ensuring net is properly oriented.

C.	Equip both standards with sliding adjustment top collars with pulley sheaves for passage of net cable.

D.	Provide field applied decals to clearly indicate net height settings to accommodate tennis, badminton, and volleyball for all uses from elementary school to international competition:

1.	Tennis: [42 inches.] [1067 mm.]

2.	Badminton: [61 inches.] [1549 mm.]

3.	Volleyball: [73, 88-1/8, and 95-5/8 inches.] [1854, 2238, 2429 mm.]

E.	Tensioning winch: Provide one standard with a heavy-duty, self-locking worm-gear mechanism. Provide opposite standard with adjustable cable anchor collar.

1.	Equip winch with [2 inches] [51 mm] wide, high tensile nylon strap with sling ring and spring-hook for connection to net cable.

2.	Winch operated by folding handle.

2.9	PORTABLE STANDARDS

***** Draper, Inc. manufactures a portable, wheeled game standard designed to attach to anchors inserted in gymnasium floor. Model PTVS-01 is constructed with a 3-1/2 inches (89 m) diameter aluminum tube. *****

A.	Type: Fixed length, portable, game standard with triangular wheeled base and designed for installation in floor anchors; PTVS-01 Portable Game Standard as manufactured by Draper, Inc.

B.	Standard post: [3-1/2 inches] [89 mm] diameter extruded 6063-T6 aluminum with clear anodized finish. Upright is custom designed to reduce weight while providing excellent rigidity. Wall Thickness if upright tube varies from [0.216 inches] [5.5 mm] to [0.433 inches] [11 mm] with thicker walls positioned to reduce deflection. Post features an indexing channel to prevent sliding collard from rotating and ensuring net is properly oriented

C.	Equip both standards with sliding adjustment top collars with pulley sheaves for passage of net cable.

D.	Base: T shaped, [28 by 28 inches] [711 by 711 mm] overall size with spring loaded handwheel and stud assembly for securing standard to floor anchors. Equip base with two non-marking wheels for transporting and three rubber pads to protect floor.

E.	Provide field applied decals to clearly indicate net height settings to accommodate tennis, badminton, and volleyball for all uses from elementary school to international competition:

1.	Tennis: [42 inches.] [1067 mm.]

2.	Badminton: [61 inches.] [1549 mm.]

3.	Volleyball: [73, 88-1/8, and 95-5/8 inches.] [1854, 2238, 2429 mm.]

F.	Tensioning winch: Provide one standard with a heavy-duty, self-locking worm-gear mechanism. Provide opposite standard with adjustable cable anchor collar.

1.	Equip winch with [2 inches] [51 mm] wide, high tensile nylon strap with sling ring and spring-hook for connection to net cable.

4.	Winch operated by folding handle.

G.	Finish:

1.	Aluminum post: Clear anodized.

2.	Steel base: High-gloss black powder coat.

2.10	VOLLEYBALL NET

A.	Type: Fabricated from high quality [4 inches] [102 mm] square mesh made with 2.5 mm black knotless nylon; Net 500004 as manufactured by Draper, Inc.

B.	Size: [32 feet] [9.8 m] long by [39-3/8 inches] [1000 mm] high.

C.	Provide double stitched, vinyl coated polyester hem around perimeter of net. At net ends, provide hem with pocket containing [1/2 inch] [133 mm] diameter fiberglass dowel.

D.	Net cable: [1/8 inch] [3 mm] diameter, [2000 pounds] [907 kg] minimum breaking strength, galvanized aircraft cable with nylon coating. Equip ends with loops formed with heavy swaged type fittings. Run cable through top hem.

E.	Rope tensioner: Provide bottom hem with [1/4 inch] [6 mm] braided nylon rope and ratchet style tensioner.

F.	Provide each net end with three [1 inch] [25 mm] wide polypropylene tension straps with buckles for tightening net.

G.	Combination Boundary Markers and Antennae: Pair of [2 inches] [51 mm] wide, white, polyester reinforced vinyl strips to attach with snap fasteners to ends of net at boundary lines; Pair of [3/8 inch] [100 mm] diameter fiberglass rods with red and white strips to extend above net [32-1/2 inches] [826mm]; 500016 Boundary Markers and Antenna as manufactured by Draper, Inc.

2.11	JUDGES STAND

A.	Type: [Movable] [Attached] platform to attach to standard with lock pins for use by official referring volleyball game; Judges Stand 5013XX as manufactured by Draper, Inc.

B.	Construction: Welded from [1-1/2 inches] [38 mm] square [14 gage] [2 mm] steel tubing and equipped with 2 non-marking wheels.

***** Standard padding color is dark blue. Draper, Inc. provides 13 additional optional colors. *****

C.	Provide stand with vinyl covered foam protection pads. Vinyl color shall be [Dark Blue] [Marine Blue] [White] [Red] [Beige] [Orange] [Yellow] [Gray] [Maroon] [Black] [Purple] [Forest Green] [Kelly Green] [Navy Blue].

2.12	ACCESSORIES

A.	Transporter: Heavy-duty unit with swivel casters designed to store and transport standards, net, protection pads, and other components and accessories required for setting up volleyball court; Volleyball Transporter 501016 as manufactured by Draper, Inc.

B.	Wall storage hooks: Pair of steel hooks mounted to wall with flat head screws to provide means for storing volleyball standards; Wall Hooks 501017 as manufactured by Draper, Inc.

C.	Vertical storage rack: Pair of steel vertical storage brackets constructed of [1/4 inch] flat steel and [3/16 inch] thick angle. Posts secured with nylon straps with hook and loop fastener. Vertical Storage Rack 501015 as manufactured by Draper, Inc.

D.	Protection pads: Removable, rectangular protection pads with hinged corners to be attached to standards to protect players; Protection Pads 5011XX as manufactured by Draper, Inc.

1.	Sizes: 5011XX: Designed to fit around standard with tensioning winch: [6 feet] [1829 mm] high by [10-1/2 inches] [267 mm] by [6-1/2 inches] [165 mm].

2.	Foam: [1-1/4 inches] [32 mm] thick Ethafoam.

3.	Cover: [14 ounces] [397 grams], polyester reinforced vinyl.

***** Standard pad color is dark blue. Draper, Inc. provides 14 additional color options. *****

4.	Vinyl color: [Dark blue (standard)] [Marine blue] [White] [Red] [Beige] [Orange] [Yellow] [Gray] [Maroon] [Black] [Purple] [Forest green] [Kelly Green] [Navy Blue].

5.	Provide each pad with 3 hook and loop fastening straps.

PART 3 - EXECUTION

***** For large projects with multiple volleyball courts it may be necessary to provide a schedule of the required items of volleyball equipment and the locations where they will be installed. This equipment schedule may be included on the drawings or in this specification section. To avoid potential conflicts, the schedule should not be included in both documents. *****

3.1	VOLLEYBALL EQUIPMENT SCHEDULE

[A.	Provide and install equipment items as listed in Volleyball Equipment Schedule on Drawings.]

[A.	Provide the following items and install in [Gymnasium] [_____], Room No. [_____]:

1.	[_____] floor sleeves with covers.

2.	[_____] floor anchors.

3.	[_____] outside telescoping standards [with protection pads].

4.	[_____] center telescoping standards [with protection pads].

5.	[_____] fixed height standards [with protection pads].

6.	[_____] portable standards.

7.	[_____] tensioning winches.

8.	[_____] volleyball nets with cable, rope tensioner, and attachment hardware.

9.	[_____] pairs of boundary markers.

10.	[_____] pairs of antennae.

11.	[_____] judges stand and protective pads.

12.	[_____] equipment transporters.

13.	[_____] pairs of wall storage hooks.]

3.2	COORDINATION

A.	Coordinate layout of volleyball courts and location of floor [sleeves] [anchors] with installation of floor surfacing and application of game lines and boundaries.

***** Include the following two paragraphs if floor sleeves are being installed to accommodate telescoping or fixed length standards. *****

B.	Coordinate location of sleeves and required size of sleeve footing with trade responsible for placing concrete. Provide sleeves in adequate time to allow casting in concrete floor slabs. Ensure that setting of sleeve compensates for type of floor finish to be provided.

C.	Ensure that sleeves for each volleyball court are spaced at [36'-0"] [10.97 m] on center.

3.3	INSTALLATION

***** Include the following paragraph if floor anchors are being installed for securing portable standards. *****

A.	Floor anchors: Drill [1-1/4 inches] [32 mm] diameter hole in concrete slab for each anchor. Accurately locate anchors based on court layout and approved shop drawings. Set anchor with fast setting grout. Ensure anchor is flush with floor finish. Let anchor set one week before using.

***** Include the following paragraph if floor sleeves and cover plates are being installed to accommodate telescoping or fixed length standards. *****

B.	Floor cover plates: Install centered directly over floor sleeves in accordance with manufacturer's instructions. Route out floor to ensure cover is flush with finished floor. Install cover with flat head screws.

***** If wall storage hooks are required, include the following paragraph. *****

C.	Wall storage hooks: Rigidly install at locations indicated on Drawings with anchors and fasteners adequate for type of substrate. Ensure pairs of hooks are aligned horizontally and spacing does not exceed length of standard.

3.4	FIELD QUALITY CONTROL

A.	[Insert standards in sleeves] [Secure portable standards to floor anchors] and attach nets, boundary markers, antennae, judge’s platform, protection padding, and other accessories. Verify that all items have been provided and are as required for complete installation.

B.	Verify that standards are vertical and rigid. [Operate telescoping feature.] Verify net height settings are accurate.

C.	Provide missing items and correct deficiencies.

3.5	CLEANING AND DEMONSTRATION

A.	Remove protective wrappings and wash surfaces.

B.	Review installed volleyball system with Owner's designated representatives. Demonstrate installation of equipment and operational features.

END OF SECTION